智能仿核信号发生器核心源码片段

队伍：摆烂小分队

时间：2026年10月22日

第1章  STM32 部分

1.1  device.c：第104–209行

设备轮询、参数COMMIT、START、STOP与错误状态处理是主控侧的核心。

104  void device_poll(void)

105  {

106      static uint32_t last_poll;

107      g_device.power_good = board_power_good();

108      g_device.board_a_present = board_a_present();

109      g_device.board_b_present = board_b_present();

110      if (board_stop_pressed()) device_stop();

111      if (g_device.state == DEVICE_RUNNING && board_millis() - g_device.last_host_ms > PC_LINK_TIMEOUT_MS) {

112          device_raise_error(DEV_ERR_TRANSFER);

113      }

114      if (!g_device.power_good) device_raise_error(DEV_ERR_POWER);

115      if ((!g_device.board_a_present || !g_device.board_b_present) && g_device.state == DEVICE_RUNNING) device_raise_error(DEV_ERR_BOARD_MISSING);

116

117      if ((board_millis() - last_poll) >= FPGA_POLL_PERIOD_MS) {

118          fpga_status_t status;

119          last_poll = board_millis();

120          if (g_device.fpga_online && fpga_link_status(&status)) {

121              memcpy(g_device.clip_count, status.clip_count, sizeof(status.clip_count));

122              memcpy(g_device.event_count, status.event_count, sizeof(status.event_count));

123              memcpy(g_device.pileup_overflow, status.pileup_overflow, sizeof(status.pileup_overflow));

124              if (status.error_flags != 0u) device_raise_error(DEV_ERR_FPGA);

125  #if BASIC_SPI_BENCH

126              if (g_device.state == DEVICE_RUNNING && status.active_generation != g_device.config_generation) device_raise_error(DEV_ERR_FPGA);

127  #endif

128          } else if (g_device.fpga_online) {

129              g_device.fpga_online = false;

130              device_raise_error(DEV_ERR_FPGA);

131          }

132      }

133  }

134

135  bool device_set_channel(uint8_t channel, const channel_config_t *config)

136  {

137      if ((channel >= OUTPUT_CHANNELS) || !validate(config)) return false;

138      g_device.shadow[channel] = *config;

139      return true;

140  }

141

142  bool device_commit(void)

143  {

144      if (g_device.state == DEVICE_RUNNING || g_device.state == DEVICE_ERROR) return false;

145      for (uint8_t ch=0; ch<OUTPUT_CHANNELS; ++ch) if (!validate(&g_device.shadow[ch])) return false;

146  #if NUCLEAR_BENCH

147      if(g_device.shadow[0].waveform==3u && !g_device.transfer_valid) return false;

148  #endif

149      uint32_t generation = g_device.config_generation + 1u;

150      bool written = true;

151      for (uint8_t ch=0; ch<OUTPUT_CHANNELS; ++ch) written = written && fpga_link_write_channel(ch, &g_device.shadow[ch]);

152      if (!written || !fpga_link_commit(generation)) {

153          device_raise_error(DEV_ERR_FPGA);

154          return false;

155      }

156  #if BASIC_SPI_BENCH

157      g_device.transfer_valid = true;

158  #endif

159      memcpy(g_device.active, g_device.shadow, sizeof(g_device.active));

160      g_device.config_generation = generation;

161      g_device.state = DEVICE_CONFIGURED;

162      return true;

163  }

164

165  bool device_start(void)

166  {

167      if (g_device.state != DEVICE_CONFIGURED || !g_device.fpga_online || !g_device.power_good ||

168          !g_device.board_a_present || !g_device.board_b_present || !g_device.transfer_valid) return false;

169      if (!fpga_link_start()) { device_raise_error(DEV_ERR_FPGA); return false; }

170      g_device.state = DEVICE_RUNNING;

171      return true;

172  }

173

174  bool device_select_preset(uint8_t preset_id, const char *name)

175  {

176  #if BASIC_SPI_BENCH

177      (void)preset_id; (void)name;

178      return false;

179  #else

180      if (preset_id >= 3u || name == NULL || g_device.state == DEVICE_RUNNING ||

181          g_device.state == DEVICE_ERROR || !fpga_link_select_preset(preset_id)) return false;

182      size_t length = strlen(name);

183      if (length >= sizeof(g_device.source_name)) length = sizeof(g_device.source_name) - 1u;

184      memcpy(g_device.source_name, name, length);

185      g_device.source_name[length] = '\0';

186      g_device.transfer_valid = true;

187      return true;

188  #endif

189  }

190

191  void device_stop(void)

192  {

193      if (!fpga_link_stop()) {

194          g_device.fpga_online = false;

195          g_device.error = DEV_ERR_FPGA;

196          g_device.state = DEVICE_ERROR;

197          return;

198      }

199      if (g_device.state != DEVICE_ERROR) g_device.state = DEVICE_IDLE;

200  }

201

202  void device_raise_error(device_error_t error)

203  {

204      (void)fpga_link_stop();

205      g_device.error = error;

206      g_device.state = DEVICE_ERROR;

207  }

208

209  bool device_clear_error(void)

1.2  fpga_link.c：第200–275行

MCU向FPGA写入单通道参数并确认generation、启停的控制链路。

200  bool fpga_link_write_channel(uint8_t channel, const channel_config_t *config)

201  {

202  #if BASIC_SPI_BENCH

203      if (channel != 0u || config == NULL) return false;

204      uint8_t base = (uint8_t)(config->baseline_v * BENCH_CODE_SCALE + 0.5f);

205      uint8_t amp = (uint8_t)(config->amplitude_v * BENCH_CODE_SCALE + 0.5f);

206      uint16_t period = (uint16_t)(config->interval_us + 0.5f);

207      uint16_t alpha = (uint16_t)(expf(-1.0f / config->decay_us) * 65536.0f + 0.5f);

208      uint8_t wire[8] = {config->enabled, base, amp, 0u, (uint8_t)period, (uint8_t)(period >> 8),

209                         (uint8_t)alpha, (uint8_t)(alpha >> 8)};

210      if (!transaction(FPGA_CMD_WRITE, FPGA_REG_CH1, wire, NULL, sizeof(wire))) return false;

211  #if NUCLEAR_BENCH

212      uint32_t probability=(uint32_t)(4294967296.0/(double)period+0.5);

213      uint8_t ext[12]={config->arrival_mode,(uint8_t)config->noise_percent,config->waveform==3u,0};

214  #if COMPETITION_BENCH

215      ext[3]=config->noise_mode | (config->pileup_mode<<1);

216  #endif

217      memcpy(ext+4,&probability,4);memcpy(ext+8,&config->seed,4);

218      if(!transaction(FPGA_CMD_WRITE,0x0110u,ext,NULL,12u)) return false;

219      memcpy(s_nuclear_wire,ext,12u);

220  #endif

221      memcpy(s_bench_wire, wire, sizeof(wire));

222      return true;

223  #else

224      fpga_channel_wire_t wire;

225      if ((channel > 1u) || (config == NULL)) return false;

226      memcpy(&wire, config, sizeof(wire));

227      return transaction(FPGA_CMD_WRITE, channel == 0u ? FPGA_REG_CH1 : FPGA_REG_CH2,

228                         (const uint8_t *)&wire, NULL, sizeof(wire));

229  #endif

230  }

231

232  bool fpga_link_commit(uint32_t generation)

233  {

234      if (!transaction(FPGA_CMD_WRITE, FPGA_REG_GENERATION, (const uint8_t *)&generation, NULL, 4u)) return false;

235      uint32_t command = 1u;

236      if (!transaction(FPGA_CMD_WRITE, FPGA_REG_CONTROL, (const uint8_t *)&command, NULL, 4u)) return false;

237  #if BASIC_SPI_BENCH

238      fpga_status_t status;

239      uint8_t active[8];

240  #if NUCLEAR_BENCH

241      uint8_t ext[12];

242      if(!transaction(FPGA_CMD_READ,0x0120u,NULL,ext,12u) || memcmp(ext,s_nuclear_wire,12u)!=0) return false;

243  #endif

244      return fpga_link_status(&status) && status.active_generation == generation && status.error_flags == 0u &&

245             transaction(FPGA_CMD_READ, 0x0108u, NULL, active, sizeof(active)) &&

246             memcmp(active, s_bench_wire, sizeof(active)) == 0;

247  #else

248      return true;

249  #endif

250  }

251

252  bool fpga_link_start(void)

253  {

254      uint32_t command = 2u;

255      if (!transaction(FPGA_CMD_WRITE, FPGA_REG_CONTROL, (const uint8_t *)&command, NULL, 4u)) return false;

256  #if BASIC_SPI_BENCH

257      uint32_t flags;

258      return transaction(FPGA_CMD_READ, 0x0048u, NULL, (uint8_t *)&flags, 4u) && (flags & 3u) == 3u;

259  #else

260      return true;

261  #endif

262  }

263

264  bool fpga_link_stop(void)

265  {

266      uint32_t command = 4u;

267      if (!transaction(FPGA_CMD_WRITE, FPGA_REG_CONTROL, (const uint8_t *)&command, NULL, 4u)) return false;

268  #if BASIC_SPI_BENCH

269      uint32_t flags;

270      return transaction(FPGA_CMD_READ, 0x0048u, NULL, (uint8_t *)&flags, 4u) && !(flags & 1u);

271  #else

272      return true;

273  #endif

274  }

275

第2章  FPGA 部分

2.1  g1_dac_spi.v：第1–55行

顶层端口、运行状态、事件与指数幅度的数据路径。

1  `timescale 1ns/1ps

2  // Bench protocol V2: all logic clocked by the board's 50 MHz oscillator.

3  module g1_dac_spi #(parameter WATCHDOG_CYCLES=12500000)(

4   input wire sys_clk, input wire key_stop_n, input wire reset_n,

5   input wire spi_cs_n, input wire spi_sck, input wire spi_mosi,

6   output wire spi_miso, output wire ready, output wire irq,

7   output reg [7:0] dac_data=16, output reg dac_clk=0);

8   reg [8:0] startup=0;

9   reg [2:0] cs=7, sck=0;

10   reg [1:0] mosi=0, rst=0, key_stop=3;

11   wire reset=!startup[8] || !rst[1] || !key_stop[1];

12   always @(posedge sys_clk) begin

13    if (!startup[8]) startup<=startup+1'b1;

14    cs<={cs[1:0],spi_cs_n}; sck<={sck[1:0],spi_sck};

15    mosi<={mosi[0],spi_mosi}; rst<={rst[0],reset_n}; key_stop<={key_stop[0],key_stop_n};

16   end

17   reg [7:0] shadow_base=16, shadow_amp=224, active_base=16, active_amp=224;

18   reg shadow_enable=1, active_enable=1;

19   reg [15:0] shadow_period=1000, active_period=1000;

20   reg [15:0] shadow_alpha=60296, active_alpha=60296;

21   reg [31:0] pending_generation=0, generation=0, event_count=0;

22   reg running=0, committed=0, link_error=0;

23   reg [31:0] watchdog=0;

24   reg [5:0] phase=0;

25   reg [15:0] sample_index=0;

26   reg [23:0] level=0;

27   reg shadow_random=0, active_random=0, shadow_spectrum=0, active_spectrum=0;

28   reg [5:0] shadow_noise=0, active_noise=0;

29   reg [1:0] shadow_options=0, active_options=0; // bit0 Gaussian, bit1 fixed pileup

30   reg [31:0] shadow_probability=4294967, active_probability=4294967;

31   reg [31:0] shadow_seed=1, active_seed=1;

32   reg [31:0] rng_event=1, rng_amp=32'h6a09e667, rng_noise=32'hbb67ae85;

33   reg [7:0] spectrum[0:255];

34   reg [255:0] table_written=0;

35   reg [31:0] clip_count=0;

36   function [31:0] advance;

37    input [31:0] x;

38    reg [31:0] y;

39    begin y=x^(x<<13); y=y^(y>>17); advance=y^(y<<5); end

40   endfunction

41   wire [31:0] random_event=advance(rng_event), random_amp=advance(rng_amp), random_noise=advance(rng_noise);

42   reg [31:0] sampled_event=0;

43   always @(posedge sys_clk) sampled_event<=random_event;

44   wire next_fire=active_random ? sampled_event<active_probability : sample_index==0;

45   reg fire=0;

46   reg [15:0] amp_product=0;

47   reg [7:0] sampled_amp=0;

48   reg [39:0] sampled_decay=0;

49   wire [16:0] amp_round={1'b0,amp_product}+17'd128;

50   wire [16:0] amp_corrected=amp_round+(amp_round>>8);

51   wire [7:0] event_amp=active_spectrum ? sampled_amp : active_amp;

52   wire [24:0] accumulated={1'b0,level}+(fire ? {1'b0,event_amp,16'b0} : 25'd0);

53   wire [23:0] limited_level=accumulated>25'h0ff0000 ? 24'hff0000 : accumulated[23:0];

54   wire [39:0] nuclear_decay=limited_level*active_alpha;

55   wire [15:0] noise_product=random_noise[15:8]*({2'b0,active_noise,1'b0}+9'd1);

2.2  g1_dac_spi.v：第1221–1263行

复位、运行更新、事件/削顶计数及自主看门狗；高斯1024项常量表不在此重复粘贴。

1221   always @(posedge sys_clk) begin

1222    if(reset) begin

1223     bit_count<=0; pos<=0; crc<=32'hffffffff; rx_shift<=0; tx_shift<=0; next_tx<=0;

1224     miso_bit<=0; frame_ok<=0; crc_received<=0; command<=0; address<=0; length<=0;

1225     shadow_base<=16; shadow_amp<=224; shadow_period<=1000; shadow_alpha<=60296; shadow_enable<=1;

1226     active_base<=16; active_amp<=224; active_period<=1000; active_alpha<=60296; active_enable<=1;

1227     pending_generation<=0; generation<=0; event_count<=0;

1228     running<=0; committed<=0; link_error<=0; watchdog<=0;

1229     phase<=0; sample_index<=0; level<=0; dac_data<=16; dac_clk<=0;

1230     read_snapshot<=0;

1231     shadow_random<=0;active_random<=0;shadow_spectrum<=0;active_spectrum<=0;

1232     shadow_options<=0;active_options<=0;

1233     shadow_noise<=0;active_noise<=0;shadow_probability<=4294967;active_probability<=4294967;

1234     shadow_seed<=1;active_seed<=1;rng_event<=1;rng_amp<=32'h6a09e667;rng_noise<=32'hbb67ae85;

1235     table_written<=0;clip_count<=0;

1236     for(i=0;i<64;i=i+1) payload[i]<=0;

1237    end else begin

1238     if(phase==49) phase<=0; else phase<=phase+1'b1;

1239     if(phase==25) dac_clk<=1;

1240     if(phase==0) dac_clk<=0;

1241     // Diagnostic: data changes 200 ns after falling edge, 300 ns before rising edge.

1242     if(phase==10) begin

1243      if(!running || !active_enable) begin dac_data<=active_base; sample_index<=0; level<=0; end

1244      else begin

1245       rng_event<=random_event; rng_amp<=random_amp; rng_noise<=random_noise;

1246       dac_data<=nuclear_sum<0 ? 8'd0 : nuclear_sum>255 ? 8'd255 : nuclear_sum[7:0];

1247       level<=sampled_decay[39:16];

1248       if(fire) event_count<=event_count+1'b1;

1249       if(nuclear_sum<0 || nuclear_sum>255 || accumulated>25'h0ff0000) clip_count<=clip_count+1'b1;

1250       if(sample_index==active_period-1) sample_index<=0; else sample_index<=sample_index+1'b1;

1251      end

1252     end

1253     if(running) begin

1254      if(watchdog==WATCHDOG_CYCLES-1) begin running<=0; link_error<=1; end

1255      else watchdog<=watchdog+1'b1;

1256     end else watchdog<=0;

1257     // Deasserted CS aborts a partial frame without changing any registers.

1258     if(cs[2]) begin bit_count<=0; pos<=0; crc<=32'hffffffff; crc_received<=0;

1259      rx_shift<=0; next_tx<=0; tx_shift<=0; miso_bit<=0; frame_ok<=0;

1260     end else begin

1261      if(!sck[2] && sck[1]) begin

1262       rx_shift<=received;

1263       bit_count<=bit_count+1'b1;

第3章  PC端上位机部分

3.1  bench.py：第12–98行

台架配置对象、校验、协议载荷与活动回读匹配。

12  class BenchConfig:

13      baseline_code: int = 16

14      amplitude_code: int = 224

15      period_us: int = 1000

16      tau_us: float = 12.0

17

18      nuclear: bool = False

19      random_events: bool = False

20      noise_peak_codes: int = 0

21      seed: int = 1

22      spectrum_table: bytes = b""

23      competition: bool = False

24      noise_mode: str = "uniform"

25      noise_rms_percent: int = 0

26      allow_pileup: bool = False

27

28      def validate(self) -> None:

29          if any(type(v) is not bool for v in (self.nuclear,self.competition,self.random_events,self.allow_pileup)):

30              raise ValueError("模式标志必须为布尔值")

31          if any(type(value) is not int for value in (self.baseline_code, self.amplitude_code, self.period_us)):

32              raise ValueError("基线码、幅度码及周期必须是整数")

33          if not (0 <= self.baseline_code <= 255 and 0 <= self.amplitude_code <= 255):

34              raise ValueError("DAC码必须在0–255范围内")

35          if self.baseline_code + self.amplitude_code > 255:

36              raise ValueError("基线码＋幅度码必须≤255")

37          if self.competition and not self.nuclear:

38              raise ValueError("N4需要核信号模式")

39          if not math.isfinite(self.tau_us) or not 1 <= self.tau_us <= (145 if self.competition else 100):

40              raise ValueError("τ超出当前固件范围（N4:1–145 μs；旧版:1–100 μs）")

41          minimum = 10 if self.competition and not self.random_events else 128

42          if not minimum <= self.period_us <= 10000:

43              raise ValueError(f"周期/平均间隔须为{minimum}–10000 μs")

44          if not self.random_events and not self.allow_pileup and self.period_us < 5 * self.tau_us:

45              raise ValueError("孤立脉冲模式要求周期≥5τ；N4可显式允许堆积")

46          if (self.noise_mode not in ("uniform", "gaussian") or type(self.noise_rms_percent) is not int

47                  or not 0 <= self.noise_rms_percent <= 20):

48              raise ValueError("噪声模式无效或高斯RMS不在0–20%FS")

49          if not self.competition and (self.noise_mode != "uniform" or self.noise_rms_percent or self.allow_pileup):

50              raise ValueError("高斯噪声及固定堆积需要N4固件")

51          if self.noise_mode == "gaussian" and self.noise_peak_codes:

52              raise ValueError("高斯模式不能同时设置均匀噪声峰值")

53          if self.noise_mode == "uniform" and self.noise_rms_percent:

54              raise ValueError("均匀模式不能同时设置高斯RMS")

55

56          if type(self.seed) is not int or not 1 <= self.seed <= 0xffffffff:

57              raise ValueError("随机种子必须为1–4294967295")

58          if type(self.noise_peak_codes) is not int or not 0 <= self.noise_peak_codes <= 32:

59              raise ValueError("均匀噪声峰值必须为0–32码")

60          if self.baseline_code < self.noise_peak_codes or self.baseline_code+self.amplitude_code+self.noise_peak_codes > 255:

61              raise ValueError("基线及幅度需为噪声保留上下余量")

62          if not isinstance(self.spectrum_table, bytes) or len(self.spectrum_table) not in (0, 256):

63              raise ValueError("能谱分位表必须为256字节")

64          if not self.nuclear and (self.random_events or self.noise_peak_codes or self.spectrum_table):

65              raise ValueError("随机/噪声/能谱需要N3固件")

66

67      def payload(self) -> dict:

68          self.validate()

69          result = {

70              "channel": 1, "enabled": True,

71              "dac_baseline_code": self.baseline_code,

72              "dac_amplitude_code": self.amplitude_code,

73              "mean_interval_us": self.period_us, "decay_us": self.tau_us,

74              **({"random_events": int(self.random_events), "noise_peak_codes": self.noise_peak_codes,

75                  "spectrum_enabled": int(bool(self.spectrum_table)), "seed": self.seed} if self.nuclear else {}),

76          }

77          if self.competition:

78              result.update(noise_mode=self.noise_mode, noise_rms_percent=self.noise_rms_percent,

79                            allow_pileup=int(self.allow_pileup))

80          return result

81

82      def matches(self, status: dict) -> bool:

83          return (

84              status.get("dac_baseline_code") == self.baseline_code

85              and status.get("dac_amplitude_code") == self.amplitude_code

86              and status.get("period_us") == self.period_us

87              and isinstance(status.get("decay_us"), (int, float))

88              and abs(status["decay_us"] - self.tau_us) <= 0.011

89              and (not self.nuclear or (status.get("random_events") == int(self.random_events)

90                  and status.get("noise_peak_codes") == self.noise_peak_codes

91                  and status.get("spectrum_enabled") == int(bool(self.spectrum_table))

92                  and status.get("seed") == self.seed

93                  and (not self.spectrum_table or status.get("spectrum_crc32") == zlib.crc32(self.spectrum_table))))

94              and (not self.competition or (status.get("noise_mode") == self.noise_mode

95                   and status.get("noise_rms_percent") == self.noise_rms_percent

96                   and status.get("allow_pileup") == int(self.allow_pileup)))

97          )

98

3.2  bench_worker.py：第12–67行

单一串口工作线程串行执行命令及状态轮询。

12  class BenchWorker(QThread):

13      """连接握手后唯一串口所有者：操作与200ms状态轮询串行执行。"""

14      updated = Signal(object)

15      completed = Signal(str)

16      failed = Signal(str, str)

17      closed = Signal(str)

18

19      def __init__(self, device: SerialDevice, parent=None) -> None:

20          super().__init__(parent)

21          self.device = device

22          self._requests: queue.PriorityQueue = queue.PriorityQueue()

23          self._counter = 0

24          self._shutdown = threading.Event()

25

26      def request(self, name: str, config=None) -> None:

27          self._counter += 1

28          self._requests.put((0 if name == "stop" else 1, self._counter, name, config))

29

30      def shutdown(self) -> None:

31          self._shutdown.set()

32

33      def run(self) -> None:

34          try:

35              while not self._shutdown.is_set():

36                  try:

37                      _priority, _counter, name, config = self._requests.get(timeout=0.2)

38                  except queue.Empty:

39                      name, config = "poll", None

40                  try:

41                      if name == "recover":

42                          status = self.device.bench_recover()

43                      elif name == "submit":

44                          status = self.device.bench_submit(config)

45                      elif name == "start":

46                          status = self.device.bench_start()

47                      elif name == "stop":

48                          status = self.device.bench_stop()

49                      else:

50                          status = self.device.read_bench_status()

51                      status["verified"] = self.device._bench_verified is not None

52                      self.updated.emit(status)

53                      if name != "poll":

54                          self.completed.emit(name)

55                  except Exception as error:

56                      logging.getLogger(__name__).exception("Bench operation failed: %s", name)

57                      self.device._bench_verified = None

58                      self.failed.emit(name, str(error))

59          finally:

60              message = "已确认停止并断开台架"

61              try:

62                  self.device.bench_stop()

63              except Exception as error:

64                  message = f"已断开，但停止未确认：{error}"

65              finally:

66                  self.device.disconnect()

67                  self.closed.emit(message)

3.3  measurement.py：第67–108行

示波器CSV的采样时间解释和原始测量读取入口。

67  def read_scope(path, *, time_scale=1.0, voltage_scale=1.0, probe=1.0):

68      """Hantek two-column CSV: increasing time, or repeated positive dt per row."""

69      if any(not math.isfinite(x) or x <= 0 for x in (time_scale, voltage_scale, probe)):

70          raise ValueError("时间/电压倍率和探头倍率必须为正有限值")

71      p = Path(path)

72      raw = p.read_bytes()

73      if len(raw) > 64 * 1024 * 1024:

74          raise ValueError("单文件超过64MB，请分段导入")

75      rows = []

76      for n, line in enumerate(raw.decode("utf-8-sig").splitlines(), 1):

77          if not line.strip():

78              continue

79          fields = line.rstrip(",").split(",")

80          try:

81              rows.append((float(fields[0]), float(fields[1])))

82          except (IndexError, ValueError):

83              if not rows and n == 1:

84                  continue

85              raise ValueError(f"第{n}行不是两列有效数值") from None

86      a = np.asarray(rows, dtype=float)

87      if a.ndim != 2 or len(a) < 16 or not np.isfinite(a).all():

88          raise ValueError("至少需要16个有限采样点")

89      t = a[:, 0] * time_scale

90      if np.all(t == t[0]):

91          if t[0] <= 0:

92              raise ValueError("重复时间列必须为正采样间隔")

93          dt = float(t[0]); t = np.arange(len(t)) * dt

94          interpretation = "重复正数时间列按采样间隔解释（请核对示波器采样率）"

95      else:

96          gaps = np.diff(t)

97          dt = float(np.median(gaps))

98          if dt <= 0 or not np.allclose(gaps, dt, rtol=.005, atol=dt * .0001):

99              raise ValueError("时间列须递增且等间隔")

100          t = t - t[0]

101          interpretation = "递增时间列"

102      return t, a[:, 1] * voltage_scale * probe, {

103          "file": p.name, "sha256": hashlib.sha256(raw).hexdigest(), "samples": len(t),

104          "dt_s": dt, "sample_rate_hz": 1 / dt, "interpretation": interpretation,

105          "time_scale": time_scale, "voltage_scale": voltage_scale, "probe": probe,

106      }

107

108

第4章  通讯协作部分

4.1  protocol.py：第40–101行

PC端帧封装、CRC检验及分片/粘包解析核心。

40  class Frame:

41      version: int

42      flags: Flags

43      command: int

44      sequence: int

45      payload: bytes = b""

46

47      def encode(self) -> bytes:

48          if not 0 <= self.version <= 255:

49              raise ValueError("协议版本必须在0–255范围内")

50          if len(self.payload) > MAX_PAYLOAD:

51              raise ValueError(f"单帧负载不能超过 {MAX_PAYLOAD} 字节")

52          body = HEADER.pack(self.version, int(self.flags), int(self.command), self.sequence, len(self.payload)) + self.payload

53          return SOF + body + CRC.pack(zlib.crc32(body) & 0xFFFFFFFF)

54

55      @classmethod

56      def decode(cls, data: bytes) -> "Frame":

57          minimum = len(SOF) + HEADER.size + CRC.size

58          if len(data) < minimum or data[:2] != SOF:

59              raise ValueError("帧头无效或数据不完整")

60          version, flags, command, sequence, length = HEADER.unpack_from(data, 2)

61          expected = minimum + length

62          if length > MAX_PAYLOAD or len(data) != expected:

63              raise ValueError("负载长度不匹配")

64          body = data[2:-CRC.size]

65          expected_crc = CRC.unpack_from(data, len(data) - CRC.size)[0]

66          if zlib.crc32(body) & 0xFFFFFFFF != expected_crc:

67              raise ValueError("CRC32校验失败")

68          return cls(version, Flags(flags), command, sequence, data[2 + HEADER.size : -CRC.size])

69

70

71  class FrameParser:

72      """处理USB CDC可能发生的分片、粘包和SOF前垃圾字节。"""

73

74      def __init__(self) -> None:

75          self.buffer = bytearray()

76

77      def feed(self, data: bytes) -> list[Frame]:

78          self.buffer.extend(data)

79          frames: list[Frame] = []

80          minimum = len(SOF) + HEADER.size + CRC.size

81          while len(self.buffer) >= minimum:

82              start = self.buffer.find(SOF)

83              if start < 0:

84                  del self.buffer[:-1]

85                  break

86              if start:

87                  del self.buffer[:start]

88              if len(self.buffer) < minimum:

89                  break

90              length = HEADER.unpack_from(self.buffer, 2)[4]

91              if length > MAX_PAYLOAD:

92                  del self.buffer[:2]

93                  continue

94              frame_length = minimum + length

95              if len(self.buffer) < frame_length:

96                  break

97              packet = bytes(self.buffer[:frame_length])

98              del self.buffer[:frame_length]

99              frames.append(Frame.decode(packet))

100          return frames

101

4.2  protocol.c：第353–385行

MCU端字节流接收和帧处理入口。

353  void protocol_init(void)

354  {

355      s_input_length = 0u; s_have_last = false; s_transfer_active = false;

356  }

357

358  void protocol_feed(const uint8_t *data, size_t length)

359  {

360      while (length-- != 0u) {

361          if (s_input_length < sizeof(s_input)) s_input[s_input_length++] = *data++;

362          else { memmove(s_input, s_input + 1u, sizeof(s_input) - 1u); s_input[sizeof(s_input) - 1u] = *data++; }

363      }

364  }

365

366  void protocol_poll(void)

367  {

368      while (s_input_length >= FRAME_OVERHEAD) {

369          size_t start = 0u;

370          while (start + 1u < s_input_length && !(s_input[start] == 0xA5u && s_input[start + 1u] == 0x5Au)) ++start;

371          if (start != 0u) { memmove(s_input, s_input + start, s_input_length - start); s_input_length -= start; }

372          if (s_input_length < FRAME_OVERHEAD) return;

373          uint32_t length = get_u32(s_input + 8u);

374          if (length > PROTOCOL_MAX_PAYLOAD) { memmove(s_input, s_input + 2u, s_input_length - 2u); s_input_length -= 2u; continue; }

375          size_t total = 16u + length;

376          if (s_input_length < total) return;

377          uint32_t expected = get_u32(s_input + 12u + length);

378          uint32_t actual = crc32_compute(s_input + 2u, 10u + length);

379          if (expected == actual && s_input[2] == PROTOCOL_VERSION) {

380              handle_frame(get_u16(s_input + 4u), get_u16(s_input + 6u), s_input + 12u, length);

381          }

382          memmove(s_input, s_input + total, s_input_length - total);

383          s_input_length -= total;

384      }

385  }

通讯协作代码的完整请求处理、SPI寄存器地址和设备状态结构分布于 protocol.c、fpga_link.c、device.c 以及 FPGA RTL；本节摘录说明关键接口，而非全部命令枚举。